# "Bent Pipe" repeating with two ID-1s

## Goal

Use two ID-1s connected together via ethernet to bridge two independent D-STAR links together, acting as a repeater station.

## Basic Link

First, start with the basic ID-1 link.

ANT          ANT

|            |

+-------------+      +------+     +------+    +-----------+

|192.168.1.102|------| ID-1 |     | ID-1 |----|192.168.1.1|

+-------------+      +------+     +------+    +-----------+

Now, add a second host to one of the ID-1s, using a switch, which is known to work fine.

ANT          ANT

|            |

+-------------+        +-------------+      +------+     +------+    +-----------+

|192.168.1.102|--------|   switch    |------| ID-1 |     | ID-1 |----|192.168.1.1|

+-------------+        +-------------+      +------+     +------+    +-----------+

|

+-------------+               |

|192.168.1.101|----------------

+-------------+

Next, consider the fact that ID-1 links operate in bridging mode, and are logically equivalent to a very long ethernet cable (as detailed in How D-STAR links behave from a networking perspective). Replace the ethernet cable connecting 192.168.1.101 to the switch with a second D-STAR link.

ANT <------> ANT

|            |

+-------------+        +-------------+      +------+     +------+    +-----------+

|192.168.1.102|--------|   switch    |------| ID-1 |     | ID-1 |----|192.168.1.1|

+-------------+        +-------------+      +------+     +------+    +-----------+

|

|

|

|

+------+

| ID-1 |

+------+

|

ANT

^

|

|

v

ANT

|

+------+

| ID-1 |

+------+

|

+-------------+               |

|192.168.1.101|----------------

+-------------+

The two ID-1s connected to a common switch function as a repeater, relaying network connectivity to 192.168.1.1 to the host 192.168.1.101. Note that each pair of ID-1s should be on a different frequency, to avoid interference between them. Note that if network connectivity is not required at the relay point, that host 192.168.1.102 can be removed from the picture, it is not critical to the relay setup.